# 1. Introduction

This document shows the definition of the “Tsukuba” data set for e-Passport interoperability test in Tsukuba in Japan.  There are four samples provided.

The “Tsukuba” data set is for creating the following data:

- EF.COM

- DG1

- DG2

- EF.SOD

# 2. Tsukuba Data Set

The Tsukuba data set consists of:

- EF.COM settings

- MRZ data for DG1

- JPEG image for DG2

- Settings for ISO/IEC 19794-5 structure in DG2

- Settings for ISO/IEC 7816-11 structure in DG2

- Document Security Object settings

- Document Signer private key and modulus for signing the Document security Object

- Document Signer certificate for verifying the Document Security Object

- Country Signing CA certificate for verifying the Document Signer Certificate

The value of using single quotation (‘’) is hexadecimal number.  (For example, 0x5F01 is ‘5F01’.)

## 2.1 EF.COM settings

Use the following settings:

| Tag | Length | Value |  |  |
| --- | --- | --- | --- | --- |
| ‘60’ | ‘14’ |  |  |  |
|  |  | Tag | Length | Value |
|  |  | ‘5F01’ | ‘04’ | ‘30313037’ (LDS Version No. = 1.7) |
|  |  | ‘5F36’ | ‘06’ | ‘303430303030’ (Unicode Version = 4.0.0) |
|  |  | ‘5C’ | ‘02’ | ‘6175’ (Tag List: “61”:DG1, “75”:DG2) |

## 2.2 MRZ data for DG1

The MRZ data is used to create DG1.  Use the following MRZ:

| Tag | Length | Value |  |  |
| --- | --- | --- | --- | --- |
| ‘61’ | ‘5B’ |  |  |  |
|  |  | Tag | Length | Value |
|  |  | ‘5F1F’ | ‘58’ | P<UTOGAIMU<<ICHIRO<<<<<<<<<<<<<<<<<<<<<<<<<< WG30004036UTO6007078M0511014<<<<<<<<<<<<<<06 |

Document type	： P

Issuing country	： UTO

Surname		： GAIMU

Given name	： ICHIRO

Passport No.	： WG3000403

Nationality	： UTO

Date of birth	： 600707

Sex		： M

Date of expiry	： 051101

Sample image of OCR-B MRZ in the data page:

P<UTOGAIMU<<ICHIRO<<<<<<<<<<<<<<<<<<<<<<<<<<

WG30004036UTO6007078M0511014<<<<<<<<<<<<<<06

## 2.3 JPEG image for DG2

A JPEG image is used to create DG2.  Use following images:

See attached files:

sample1:  kitamura_20k.jpg

sample2:  kitamura_34k.jpg

### DG2 settings

3 bytes size of data length is stored in each “Var.” area.  Use the following settings:

| Tag | Length | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ‘75’ | Var. |  |  |  |  |  |  |
|  |  | Tag | Length | Value |  |  |  |
|  |  | ‘7F61’ | Var. |  |  |  |  |
|  |  |  | Tag | Length | Value |  |  |
|  |  |  | ‘02’ | ‘01’ | ‘01’ (one biometrics data) |  |  |
|  |  |  | ‘7F60’ | Var. |  |  |  |
|  |  |  |  | Tag | Length | Value |  |
|  |  |  |  | ‘A1’ | ‘0E’ |  |  |
|  |  |  |  |  | Tag | Length | Value |
|  |  |  |  |  | ‘81’ | ‘01’ | ‘02’ (face) |
|  |  |  |  |  | ‘82’ | ‘01’ | ‘00’ (none) |
|  |  |  |  |  | ‘87’ | ‘02’ | ‘0101’ (ISO/IEC JTC1 SC37) |
|  |  |  |  |  | ‘88’ | “02’ | ‘0501’ (19794-5) |
|  |  |  |  | Tag | Length | Value |  |
|  |  |  |  | ‘5F2E’ | Var. |  |  |
|  |  |  |  |  |  | ‘46414300’ (Format Identifier) |  |
|  |  |  |  |  |  | ‘30313000’ (Version No.) |  |
|  |  |  |  |  |  | 4bytes size var. (Length of Record) |  |
|  |  |  |  |  |  | ‘0001’ (Number of Facial Image) |  |
|  |  |  |  |  |  | 4bytes size var. (Data Size) |  |
|  |  |  |  |  |  | ‘0000’ (Number of Feature Points) |  |
|  |  |  |  |  |  | ‘00’ (Gender) |  |
|  |  |  |  |  |  | ‘00’ (Eye Color) |  |
|  |  |  |  |  |  | ‘00’ (Hair Color) |  |
|  |  |  |  |  |  | ‘000000’ (Feature Mask) |  |
|  |  |  |  |  |  | ‘0000’ (Expression) |  |
|  |  |  |  |  |  | ‘000000’ (Pose Angle) |  |
|  |  |  |  |  |  | ‘000000’ (Pose Angle Uncertainty) |  |
|  |  |  |  |  |  | ‘02’ (Face Image Type) |  |
|  |  |  |  |  |  | ‘00’ (Image Data Type) |  |
|  |  |  |  |  |  | ‘01A4’ (Width) |  |
|  |  |  |  |  |  | ‘021C’ (Height) |  |
|  |  |  |  |  |  | ‘00’ (Image Color Space) |  |
|  |  |  |  |  |  | ‘00’ (Source Type) |  |
|  |  |  |  |  |  | ‘0000’ (Device Type) |  |
|  |  |  |  |  |  | ‘0000’ (Quality) |  |
|  |  |  |  |  |  | JPEG data |  |

### ISO/IEC 7816-11 settings (BHT)

The ISO/IEC19794-5 BLOB will be wrapped up in an ISO/IEC7816-11 structure.  Use the following settings:

| Tag | Description | Value |
| --- | --- | --- |
| ‘80’ | Patron header version number | Not used |
| ‘81’ | Biometric type | ‘02’ (face) |
| ‘82’ | Biometric subtype | ‘00’ (none) |
| ‘87’ | Format owner | ‘0101’ (ISO/IEC JTC1 SC37) |
| ‘88’ | Format type | ‘0501’ (19794-5) |

### ISO/IEC 19794-5 settings (Facial Record Header, Facial Record Data)

The JPEG image will be wrapped up in an ISO/IEC19794-5 structure.  Use the following Settings:

| Field | Value |
| --- | --- |
| Number of Feature Points | ‘0000’ |
| Gender | ‘00’ |
| Eye Color | ‘00’ |
| Hair Color | ‘00’ |
| Feature Mask | ‘000000’ |
| Expression | ‘0000’ |
| Pose Angle | ‘000000’ |
| Pose Angle Uncertainty | ‘000000’ |
| Face Image Type | ‘02’ (Full Frontal Image Format) |
| Image Data Type | ‘00’ (JPEG) |
| Width | ‘01A4’ (420 Pixels) |
| Height | ‘02C1’ (540 Pixels) |
| Image Color Space | ‘00’ |
| Source Type | ‘00’ |
| Device Type | ‘0000’ |
| Quality | ‘0000’ |

## 2.4 Document Security Object settings

The Document Security Object is a RFC3369 SignedData object.  Use the following settings:

| Field | Value |
| --- | --- |
| SignedData |  |
| version | ‘03’ |
| digestAlgorithm |  |
| algorithm | 2.16.840.1.101.3.4.2.1 (SHA2-256) |
| parameters | NULL |
| encapContentInfo |  |
| eContentType | 2.23.136.1.1.1 (LDSSecurityObject) |
| eContent | DER encoding of the LDSSecurityObject |
| certificates | Add only the Document Signer Certificate |
| crls | Not used |
| signerInfos |  |
| signerInfo |  |
| version | ‘01’ |
| sid | Use the option issuerAndSerialNumber |
| digestAlgorithm | 2.16.840.1.101.3.4.2.1 (SHA2-256) |
| signedAttrs | Use the attributes: • ContentType: OID: 1.2.840.113549.1.9.3 Value: 2.23.136.1.1.1 (LDSSecurityObject) • SigningTime: OID: 1.2.840.113549.1.9.5 Value: Signing date (UCTTime) • MessageDigest: OID: 1.2.840.113549.1.9.4 Value: SHA2-256-digest(eContent) |
| signatureAlgorithm | Use the signature algorithm • algorithm: OID: 1.2.840.113539.1.1.10 (RSA-PSS) • HashAlgorithm: OID: 2.16.840.1.101.3.4.2.1 (SHA2-256) • MaskGenAlgorithm: OID: 1.2.840.113549.1.1.8 (MGF1) HashAlgorithm = 2.16.840.1.101.3.4.2.1 (SHA2-256) • saltLength: value = 32 |
| signature | Calculated signature |
| unsignedAttrs | Not used |

## 2.5 Document Signer Certificate

The Document Signer Certificate must be stored inside the Document Security Object.  Use the following certificate:

See attached file:  Tsukuba_CDS.der

## 2.6 Country Signing CA Certificate

The Country Signing CA Certificate is needed for verifying the Document Signer Certificate.  Use the following certificate:

See attached file:  Tsukuba_CCSCA.der

# Appendix A. Dumping image of DG2 files

- kitamura_20k.dg2

- ADDRESS : +0 +1 +2 +3 +4 +5 +6 +7 +8 +9 +A +B +C +D +E +F - 00000000 : 75 82 4F 07 7F 61 82 4F 02 02 01 01 7F 60 82 4E - 00000010 : FA A1 0E 81 01 02 82 01 00 87 02 01 01 88 02 05 - 00000020 : 01 5F 2E 82 4E E5 46 41 43 00 30 31 30 00 00 00 - 00000030 : 4E E5 00 01 00 00 4E D7 00 00 00 00 00 00 00 00 - 00000040 : 00 00 00 00 00 00 00 00 02 00 01 A4 02 1C 00 00 - 00000050 : 00 00 00 00 FF D8 FF E0 00 10 4A 46 49 46 00 01 00000060 : 01 01 01 2C 01 2C 00 00 FF DB 00 43 00 0A 07 07

- kitamura_34k.dg2

- ADDRESS : +0 +1 +2 +3 +4 +5 +6 +7 +8 +9 +A +B +C +D +E +F - 00000000 : 75 82 86 72 7F 61 82 86 6D 02 01 01 7F 60 82 86 - 00000010 : 65 A1 0E 81 01 02 82 01 00 87 02 01 01 88 02 05 - 00000020 : 01 5F 2E 82 86 50 46 41 43 00 30 31 30 00 00 00 - 00000030 : 86 50 00 01 00 00 86 42 00 00 00 00 00 00 00 00 - 00000040 : 00 00 00 00 00 00 00 00 02 00 01 A4 02 1C 00 00 - 00000050 : 00 00 00 00 FF D8 FF E0 00 10 4A 46 49 46 00 01 - 00000060 : 01 01 01 2C 01 2C 00 00 FF DB 00 43 00 04 03 03